CX25858

Linux Driver

Installation and User Guide

USG-202607C

December  2011

Revision History

| Revision | Date | Description |
| --- | --- | --- |
| A | July 22, 2010 | Initial release |
| B | August 18, 2010 | Revision B release *Clarified procedures in Chapter 2. *Removed Appendix A. |
| C | December 7, 2011 | Revision C release *Updated to use media_build |

© 2010, Conexant Systems, Inc.

All Rights Reserved.

Information in this document is provided in connection with Conexant Systems, Inc. (“Conexant”) products. These materials are provided by

Conexant as a service to its customers and may be used for informational purposes only. Conexant assumes no responsibility for errors or

omissions in these materials. Conexant may make changes to this document at any time, without notice. Conexant advises all customers to

ensure that they have the latest version of this document and to verify, before placing orders, that information being relied on is current and

complete. Conexant makes no commitment to update the information and shall have no responsibility whatsoever for conflicts or

incompatibilities arising from future changes to its specifications and product descriptions.

No license, express or implied, by estoppel or otherwise, to any intellectual property rights is granted by this document. Except as provided in

Conexant’s Terms and Conditions of Sale for such products, Conexant assumes no liability whatsoever.

THESE MATERIALS ARE PROVIDED “AS IS” WITHOUT WARRANTY OF ANY KIND, EITHER EXPRESS OR IMPLIED, RELATING TO SALE

AND/OR USE OF CONEXANT PRODUCTS INCLUDING LIABILITY OR WARRANTIES RELATING TO FITNESS FOR A PARTICULAR

PURPOSE, CONSEQUENTIAL OR INCIDENTAL DAMAGES, MERCHANTABILITY, OR INFRINGEMENT OF ANY PATENT, COPYRIGHT OR

OTHER INTELLECTUAL PROPERTY RIGHT. CONEXANT FURTHER DOES NOT WARRANT THE ACCURACY OR COMPLETENESS OF

THE INFORMATION, TEXT, GRAPHICS OR OTHER ITEMS CONTAINED WITHIN THESE MATERIALS. CONEXANT SHALL NOT BE

LIABLE FOR ANY SPECIAL, INDIRECT, INCIDENTAL, OR CONSEQUENTIAL DAMAGES, INCLUDING WITHOUT LIMITATION, LOST

REVENUES OR LOST PROFITS, WHICH MAY RESULT FROM THE USE OF THESE MATERIALS.

Conexant products are not intended for use in medical, lifesaving or life sustaining applications. Conexant customers using or selling Conexant

products for use in such applications do so at their own risk and agree to fully indemnify Conexant for any damages resulting from such

improper use or sale.

The following are trademarks of Conexant Systems, Inc.: Conexant® and the Conexant C symbol. Product names or services listed in this

publication are for identification purposes only, and may be trademarks of third parties. Third-party brands and names are the property of their

respective owners.

For additional disclaimer information, please consult Conexant’s Legal Information posted at www.conexant.com which is incorporated by

reference.

Conexant Lead-free products are China RoHS Compliant:

Reader Response: Conexant strives to produce quality documentation and welcomes your feedback. Please send comments and suggestions

to tech.pubs@conexant.com. For technical questions, contact your local Conexant sales office or field applications engineer.

2 Conexant USG-202607C

Preliminary Information/Conexant Proprietary and Confidential 12/7/11

Contents

1 Overview . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 5

2 Setup and Installation . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 7

2.1 Building the V4L2 and CX25858 Modules . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 7

2.1.1 Loading the CX25858 Module . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 8

2.2 Running the CX25858 Module Scripts . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 9

2.2.1 Streaming Raw Video in D1 Resolution . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 9

2.2.2 Record H.264 Stream . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 9

USG-202607C Conexant 3

12/7/11 Preliminary Information/Conexant Proprietary and Confidential

CX25858 Installation and User Guide

4 Conexant USG-202607C

Preliminary Information/Conexant Proprietary and Confidential 12/7/11

1

Overview

This document provides an overview of the steps required to do the following:

Build the driver for CX25858 based Capture board.

Add the driver to the kernel's loadable modules so that it does not need to be

manually loaded every time the system boots.

Run scripts to stream raw video and record H.264 files.

Finally, the document summarizes the various source files present in the CX25858

driver.

USG-202607C Conexant 5

12/7/11 Preliminary Information/Conexant Proprietary and Confidential

Overview CX25858 Installation and User Guide

6 Conexant USG-202607C

Preliminary Information/Conexant Proprietary and Confidential 12/7/11

2

2.1

USG-202607C

12/7/11

Setup and Installation

Building the V4L2 and CX25858 Modules

NOTE:

You will need root privileges for these operations.

1. Download and unzip the CX25858 EVK Linux Distribution release into a directory

(e.g., /home/userxxx/cx25858). There will be two gz files listed as follows:

media_build_source_2011-11-09.tar.gz

cx25858_src_2.0.1xx.tar.gz.

media_build_source_2011-11-09.tar.gz contains the v4l2 community release that the current released

driver is synced to, and cx25858_src_2.0.1xx.tar.gz contains the CX25858 EVK

linux driver source files.

2. Open a Terminal window and ensure you are the root user by typing the following

command.

$ whoami

3. To check out the entire source, create a project directory under the user directory,

for example, “/home/userxxx/projects/” where you want to get all the source tree for

v4l2. Change to that directory.

4. Extract the v4l2 files.

$ tar -xzf media_build_source_2011-11-09.tar.gz

5. The next step is to build the V4L2 frame work.

a. Step 4 should have created a directory called media_build. Change to this directory.

b. Type yum install –y perl-Proc-ProcessTable

Type these commands if If using Ubuntu:

apt-get install libproc-processtable-perl

apt-get install ncurses-dev

c. Type make

d. Type make install

e. If you want to clean the output, do a “make clean,” and If you want to clean the

build configuration, do a “make distclean.” This is definitely required if you are

adding a new project.

f. If you are successful, move to the next step, or reboot and try again.

6. The next step is to merge in the CX25858 sources.

7. Copy the CX25858 Linux driver tarball (cx25858_src_2.0.1xx.tar.gz) to media_build

directory and untar the same. If prompted, overwrite the three files.

Conexant 7

Preliminary Information/Conexant Proprietary and Confidential

Setup and Installation CX25858 Installation and User Guide

8. Building CX25858 driver consists of two steps. The first one is a one-time

operation (all commands given from the media_build folder).

a. At the command prompt type

# make distclean (this is to be done only the first time when you are

adding a new module).

# make menuconfig

This is a one-time operation to create the proper .config files and other

supporting files in the v4l folder (under the media_build folder).

b. Select the proper modules that are required. Refer to the menuconfig document

for details.

From the menu, you need to select “Conexant 25858 Support” under the following:

- Multimedia Support

- Video Capture adapters

- V4L PCI(e) devices

c. In the command prompt type

#make

#make install

d. For subsequent clean builds

#make clean (to cleanup for subsequent full makes)

2.1.1 Loading the CX25858 Module

9. After step 4, the cx25858.o binary should be built in the v4l folder. The next step is

to load the module. To do this, do the “modprobe cx25858” command.

10. Take a look at the “dmesg” to make sure that the cx25858 library is loaded.

11. On subsequent reboots, the module should be automatically loaded at boot time

because of the menuconfig operation that was done.

8 Conexant USG-202607C

Preliminary Information/Conexant Proprietary and Confidential 12/7/11

2.2

2.2.1

2.2.2

USG-202607C

12/7/11

CX25858 Installation and User Guide Setup and Installation

Running the CX25858 Module Scripts

Make sure the video and audio sources are connected and you have logged in as

super user (or root).

Streaming Raw Video in D1 Resolution

1. The scripts are located in the media_build/linux/scripts directory.

2. Change directory to scripts.

3. In the command prompt type

#./mplay_video n (n=0 - 8)

4. The above scripts streams raw D1 video from channel n.

Record H.264 Stream

1. Go to the avcenc directory located under scripts directory.

2. In the command prompt, type #./run_atlas n m NTSC

a. Where n - is the starting codec number (0 or 1 for single board)

b. Where m is the ending codec number (0 or 1 for single board)

c. Where NTSC is for NTSC format, and for PAL enter PAL

NOTE:

Make sure n is less than or equal to m.

3. Once the system enters the command shell, type

> record_all(0)

> record_all(1)

>

This will start the H.264 recording in the all the 8 channels.

4. To stop the recording type

>stop_all(0)

>stop_all(1)

5. Once the recording is stopped, you should see 8 files with extension .qbx (one for

each channel)

6. Use the tool qbox2h4v to convert the qbx files to h.264 files. In the command

prompt, run

# ./qbox2h4v

7. The above command will generate 8 h.264 files named main1.h264, main2.h264...

main8.h264.

8. You can play individual files. To play the file, you need to use the mplayer. In the

command prompt type

# mplayer mainx.h264

(Where mainx.h264 is the name of the h.264 file)

Conexant 9

Preliminary Information/Conexant Proprietary and Confidential

Setup and Installation CX25858 Installation and User Guide

10 Conexant USG-202607C

Preliminary Information/Conexant Proprietary and Confidential 12/7/11

www.conexant.com

General Information:

U.S. and Canada: (888) 855-4562

International: (949) 483-4600

Headquarters – Newport Beach

4000 MacArthur Blvd.

Newport Beach, CA 92660